Report from Visit on February 15, 2005 in Milan

1°) Set point Program

Japan accepts modification of existing SDC15. One Step program will be available for specific SDC15 models (limited numbers) as same as SDC35/36.

So please validate SDC35 functionality, if it works, fine.

We need to define the complete model numbers requested for this application. 

The new model will be as following:

If Standard C15TV0RA0100 (for example)

Then Special model will be: C15TV0RA0100Mxxx (with xxx = Step program)

Please let me know by return which model should be selected.

Price will be ELP – 50%, no extra charge for this special but no possibility to get special lower cost.

Volume: 2000 units minimum per year (this is important point)

Delivery: not yet defined

2°) Hot Runner

Impossible

Complete redesign of SDC15 software

Capacity of memory in SDC15 not big enough

Too expensive development

3°) Motor Drive

Yes, this one is possible. Feature will be the same as SDC35/36 but without Feedback positioning option. It will be only estimated positioning.

The output could be 2 relays (OUT1 & ALM1) or two volt outputs (option “VV”) to drive power packs.

Let me know what do you recommend?

For positioning estimation, the unit will work as following: At POWER ON, close relay will keep close up to traveling time preset value. This will indicate 0% (close valve), form this point we will take the position reference for calculation of the estimated position.

Price will be ELP – 50%, no extra charge for this special but no possibility to get special lower cost.

Volume: 2000 units minimum per year

Delivery: not yet defined

4°) Universal input

Impossible

Too expensive development and big hardware impact

Could be for long term development.

